MW-MTPW-544333-CW-RFB / Gravelling, embankment formation, spot and drainage improvement on Kholoni[U1.1] - Sitolo (UD) Road (10.0 Kms) in Mchinji District.

The project involves Reshaping, spot Gravelling, embankment formation, and drainage improvement on Kholomoni - Sitolo (UD) Road (10.0 Kms) in Mchinji District.

Major Works

- Reshaping – 10 Kms;

- Construction of 2m x 1m 1 cell, and 2m x 1m 3cell box culverts;

- Embankment formation – about 80 meters; and

- Spot Gravelling.

The execution of the works should follow SATCC specifications and standards; and will utilize labour intensive construction methods.

Road Works

    - General

The existing road is undesignated road. The road passes through flat to gently sloping terrain. The road was previously reshaped to typical cross-section for UD roads. However, over time deterioration has occurred.  The carriage way has narrowed in some places; side drains have silted in some sections while other sections have become gullies; and some culverts have been fully silted.  Therefore, the interventions under this project are aimed at restoring the typical cross section of the road; raise the road in some selected spots; and improve drainage. Gravelling will be done in selected spots where the road becomes slippery when wet. All earthworks and gravelling works will involve watering and compaction.

    - Horizontal alignment

The horizontal alignment will follow the existing alignment.

    - Vertical alignment

The vertical alignment will generally be raised such that the finished road surface is generally slightly above the existing road level. Where new cross drainage structures will be constructed the road will be raised to accommodate the new structures and to ensure finished level to be above peak flood level.

Structures

A list of drainage structures to be constructed on this contract, showing location, size of openings and number of openings, is included in drawings.

Services

All known services are indicated on the drawings. It is expected that more services may be situated within the roads reserve. The Contractor must contact all service providers to check where services are situated in the construction area well in advance of the start of construction, and such services must be exposed to confirm their position and the level thereof. The Engineer may request the Contractor to move or protect the services.

Where services must be moved or protected due to construction works, these actions must be done by the Contractor in close co-operation with the service owner.