MW-MTPW-544341-CW-RFB / Gravelling, embankment formation, spot and drainage improvement on Kanyenda via Mwendayenda Scheme (UD) Road (3 Kms) in Dedza District.

The project involves embankment formation, and drainage improvement on Kanyenda via Mwendayenda Scheme (UD) Road (3 Kms) in Dedza District

Major Works

- Construction of 4m x 3m 2 cell box culvert across Mwendayenda Stream;

- Embankment formation for culvert approaches, about 100m total; and

- Erosion protection works.

Road Works

    - General

The existing road is undesignated road. The road passes through some steep slopes. The works will include spot repairs to restore the typical cross section of the road; gravelling in selected spots and construction of large box culvert across Mwendayenda stream. All earthworks and gravelling works will involve watering and compaction.

    - Horizontal alignment

The horizontal alignment will follow the existing alignment.

    - Vertical alignment

The vertical alignment will be raised to accommodate the new box culvert and to ensure finished level to be above peak flood level.

Structures

A list of drainage structures to be constructed on this contract, showing location, size of openings and number of openings, is included in drawings.

Services

The Contractor must contact all service providers to check where services are situated in the construction area well in advance of the start of construction, and such services must be exposed to confirm their position and the level thereof. The Engineer may request the Contractor to move or protect the services.

Where services must be moved or protected due to construction works, these actions must be done by the Contractor in close co-operation with the service owner.