MW-MTPW-544351-CW-RFB / Spot improvements and installation of drainage structures on Senzani - Doviko (T394 / S125) Road (18 Kms) in Ntcheu District- Section 2 (Km 24-42).

The project involves Selected spot improvements and installation of drainage structures on Senzani - Doviko (T394 / S125) Road (Section 2 -18 Kms) in Ntcheu District

Section 2 starts Km 24+000 (Jumbe) to Km 42+000 – Doviko.

Major Works

- Reshaping – about 5Kms;

- Filling deep side gullies;

- Drain lining and check dams; and

- Gravelling - about 3 Kms

The execution of the works should follow SATCC specifications and standards.

Road Works

    - General

The road passes through mountainous terrain.  The condition of the road is so varied, with some sections in good condition while other sections are very bad. There are several rock outcrops; several steep and slippery sections; Mitre drains are no longer functioning as most of them are at a higher level than the road; Side drains have silted in some sections while other sections have become deep gullies; and many culverts are fully silted.  Therefore, the interventions under this project are aimed at improving the road in the selected bad spots; improve drainage; and gravelling will be done in selected spots. All earthworks and gravelling works will involve watering and compaction.

    - Horizontal alignment

The horizontal alignment will follow the existing alignment.

    - Vertical alignment

There will be some changes to the vertical alignment in places where with the current alignment there is zero sight distance.

Structures

A list of drainage structures to be constructed on this contract, showing location, size of openings and number of openings, is included in drawings.

Services

All known services are indicated on the drawings. It is expected that more services may be situated within the roads reserve. The Contractor must contact all service providers to check where services are situated in the construction area well in advance of the start of construction, and such services must be exposed to confirm their position and the level thereof. The Engineer may request the Contractor to move or protect the services.

Where services must be moved or protected due to construction works, these actions must be done by the Contractor in close co-operation with the service owner.