MW-MTPW-544353-CW-RFB / Selected spot improvements and construction of box culverts on Mdeka - Chinyangute (UD) Feeder Roads (10.6 Kms) in Blantyre District.

The project involves Selected spot improvements and construction of box culverts on Mdeka - Chinyangute (UD) Feeder Roads (10.6 Kms) in Blantyre District. The Feeder Roads consist of two branches: Mdeka – Namalondwe School (5.5 Kms) and Mdeka – Mpanda School (5.1 Kms)

Major Works

- Drainage structures repairs and erosion protection works along Mdeka – Chinyangute;

- Reshaping Mdeka – Mpanda, about 5.1 Kms; and

- Gravelling, about 2 Kms.

The execution of the works should follow SATCC specifications and standards; and will utilize labour intensive construction methods.

Road Works

    - General

The existing road is undesignated road. The road passes through rolling terrain. The works will include spot repairs to restore the typical cross section of the road; gravelling in selected spots and construction and repairs of culverts. All earthworks and gravelling works will involve watering and compaction.

    - Horizontal alignment

The horizontal alignment will follow the existing alignment.

    - Vertical alignment

The vertical alignment will be raised in some sections to accommodate the new drainage structures and to ensure finished level to be above peak flood level.

Structures

A list of drainage structures to be constructed on this contract, showing location, size of openings and number of openings, is included in drawings.

Services

The Contractor must contact all service providers to check where services are situated in the construction area well in advance of the start of construction, and such services must be exposed to confirm their position and the level thereof. The Engineer may request the Contractor to move or protect the services.

Where services must be moved or protected due to construction works, these actions must be done by the Contractor in close co-operation with the service owner.