MW-MTPW-544347-CW-RFB / Selected spot improvements and installation of drainage installation structures on Ndege-Govala- Mpyupyu-Kachulu (S143) road in Zomba District

The project involves Selected spot improvements and installation of drainage structures on Ndege – Govala – Mpyupyu - Kachulu (S143) Road (14 Kms) in Zomba District

Major Works

- Construction of 1.5m x 1m 2 cell, and 1.5m x 1m 1 cell box culverts;

- Embankment formation – about 4 kms; and

- Reinstatement of mitre drains.

The execution of the works should follow SATCC specifications and standards.

Road Works

    - General

The road passes through flat to gently sloping terrain.  Due to several years of grading out the road level is below natural ground level, making it become a water way when it rains; Mitre drains are no longer functioning as most of them are at a higher level than the road; Side drains have silted in some sections while other sections have become gullies; and many culverts are fully silted.  Therefore, the interventions under this project are aimed at raising the road in some selected spots; improve drainage; and gravelling will be done in selected spots. All earthworks and gravelling works will involve watering and compaction.

    - Horizontal alignment

The horizontal alignment will follow the existing alignment.

    - Vertical alignment

The vertical alignment will be raised in some critical sections to facilitate functionality of side drains. Where new cross drainage structures will be constructed the road will be raised to accommodate the new structures.

Structures

A list of drainage structures to be constructed on this contract, showing location, size of openings and number of openings, is included in drawings.

Services

There are water pipes along and crossing the road. It is expected that more services may be situated within the roads reserve. The Contractor must contact all service providers and farm owners to check where services are situated in the construction area well in advance of the start of construction, and such services must be exposed to confirm their position and the level thereof. The Engineer may request the Contractor to move or protect the services.

Where services must be moved or protected due to construction works, these actions must be done by the Contractor in close co-operation with the service owner.